Upgrade procedures:
1.) Download the zip package from the link above

2.) Extract this package to the root directory (not inside a folder) of a USB thumb drive - FAT32 formatted
There should be 6 files in your thumb drive's root directory when it's done.

3.) Plug this USB drive containing the 6 upgrade files to your Z83-4, then powering it up and press "F11" on your keyboard during boot to enter the boot device menu

4.) Select "Built-in UEFI Shell" when you see the below boot device menu and hit "Enter" on your keyboard

5.) Type "fs3:" and then hit "Enter" to read from the USB drive
(you may also type "dir" and hit "Enter" after this step to verify all the ugrade files are there)

6.) Type "afuefix64.efi CHT0A180.bin /p /b /n /x” and hit "Enter" to execute the upgrade package, it shall take a few minutes and upon completion,

Patiently wait for the upgrade to complete, DO NOT disconnect the power source during the process you may brick your machine!

7.) then type exit to restart PC.