1. Extract the compressed file to a folder on a Windows based PC, you will get 2 folders:

- ProvisioningTool20170527

- 1.4_2.2hdcp-key

2. Go to “ProvisioningTool20170526” folder, and double-click ‘ProvisioningTool.exe’ to run this flashing tool.

3. Use a USB-A (connect to Windows based PC) to USB-C (connect to X39) cable to connect the PC and the X39.

4. Use a paper-clip to press the discovery button (beside the antenna), you can feel a distinct resistance, hold it, do NOT release.

Then press and hold the “POWER” button (beside the TF slot) for around 3 seconds, until the “ProvisioningTool” recognize the NEO X39, and says “Found one loader” at the bottom.

5. Click the “Hdcp” option, then there’s a window come out.

6. (1) Check “HDCP1.4 HDMI” box and “HDCP2.X WFD” box.

(2) Separately click ‘1.4 Key File’ and ‘2.X Key File’ to load the relating key file.

(3) Then click “Write Key” to write the HDCP keys.

7. The flashing process shall be finish in a second.

Then the X39 shall automatically power on, and the key is successfully flashed into the device.